Computer Networks (CSCI 5550/ CSCI 4550/ ECE 4245)

Exam- 3 (V-2)

Marks: 70, Total Time: 35 Mins

Name:__________________                                    Banner  Id:_________________

- (5 Points) What is the role of the forwarding table within a router?

- (5 points) What is HOL blocking? Does it occur in input ports or output ports?

- (5 Points) What is the difference between forwarding and routing in the network layer?

- (10 points) What are the challenges in transitioning from IPv4 to IPv6 in the public Internet, and what approach has been most widely adopted?

- (10 points) What is subnetting and how does it work?

- (15 points) What are the different queuing disciplines used in routers, and how do they determine the order in which queued packets are transmitted over an outgoing link?

- (20 Points) Consider a datagram network using 32-bit host addresses. Suppose a router has four links, numbered 0 through 3, and packets are to be forwarded to the link interfaces as follows:

| Destination Address Range | Link Interface |
| --- | --- |
| 11100000 00000000 00000000 00000000 through 11100000 00111111 11111111 11111111 | 0 |
| 11100000 01000000 00000000 00000000 through 11100000 01000000 11111111 11111111 | 1 |
| 11100000 01000001 00000000 00000000 through 11100001 01111111 11111111 11111111 | 2 |
| otherwise | 3 |

a. Provide a forwarding table that has five entries, uses longest prefix matching, and forwards packets to the correct link interfaces.

b. Describe how your forwarding table determines the appropriate link interface for datagrams with destination addresses:

11001000 10010001 01010001 01010101

11100001 01000000 11000011 00111100

11100001 10000000 00010001 01110111

Computer Networks (CSCI 5550/ CSCI 4550/ ECE 4245)

Exam- 3 (V-2)

MCQS and True or False

Time : 15 mins, Marks: 20

Name:__________________                                    Banner  Id:_________________

- (2 Points) What are the two important network-layer functions?

a. Forwarding and switching

b. Routing and switching

c. Forwarding and routing

d. Broadcasting and multicasting

- (2 Points) What is the purpose of the service model provided by the network layer?

a) To define the characteristics of end-to-end delivery of packets between sending and receiving hosts

b) To define the characteristics of data transmission at the physical layer

c) To define the characteristics of packet transmission at the transport layer

d) To define the characteristics of packet loss at the data link layer

- (2 Points) What is the difference between an input/output port and a software port?

A) Input/output ports are associated with network applications and sockets, while software ports terminate incoming physical links at a router

B) Input/output ports terminate incoming physical links at a router, while software ports are used by the routing processor to communicate with the remote controller

C) Input/output ports are physically implemented, while software ports are virtually implemented

D) There is no difference

- (2 Points) Which of the following is not a switching method used by routers?

a. Switching via memory

b. Switching via a bus

c. Switching via an interconnection network

d. Switching via a hub

- (2 Points) Which queuing discipline prioritizes packets based on their priority class?

a) First-Come-First-Served (FCFS)

b) Priority queuing

c) Round-robin queuing

d) Weighted fair queuing

True/False:

6.   (2 points ) The routing function is responsible for moving packets from a sending host to a receiving host.

7.  (2 points ) The routing processor is responsible for computing the forwarding table for the router.

8.  (2 points ) Under the FIFO queuing discipline, packets leave the queue in the same order in which they arrived.

9.  (2 points ) Can IPv4-capable systems handle IPv6 datagrams without modifications?

10.  (2 points) Head-of-the-line (HOL) blocking can occur in an input-queued switch when two packets at the front of two input queues are destined for the same output queue.