Computer Networks (CSCI 5550/ CSCI 4550/ ECE 4245)

Exam- 2 (V-2)

Marks: 55, Total Time: 35 Mins

Name:__________________                                    Banner  Id:_________________

- (5 Points) What is the role of the forwarding table within a router?

- (5 points) What is HOL blocking? Does it occur in input ports or output ports?

- (5 Points) What is the difference between forwarding and routing in the network layer?

- (10 points) What are the challenges in transitioning from IPv4 to IPv6 in the public Internet, and what approach has been most widely adopted?

- (10 points) What is subnetting and how does it work?

- (15 points) What are the different queuing disciplines used in routers, and how do they determine the order in which queued packets are transmitted over an outgoing link?

- (20 Points) Consider a datagram network using 32-bit host addresses. Suppose a router has four links, numbered 0 through 3, and packets are to be forwarded to the link interfaces as follows:

| Destination Address Range | Link Interface |
| --- | --- |
| 11100000 00000000 00000000 00000000 through 11100000 00111111 11111111 11111111 | 0 |
| 11100000 01000000 00000000 00000000 through 11100000 01000000 11111111 11111111 | 1 |
| 11100000 01000001 00000000 00000000 through 11100001 01111111 11111111 11111111 | 2 |
| otherwise | 3 |

a. Provide a forwarding table that has five entries, uses longest prefix matching, and forwards packets to the correct link interfaces.

b. Describe how your forwarding table determines the appropriate link interface for datagrams with destination addresses:

11001000 10010001 01010001 01010101

11100001 01000000 11000011 00111100

11100001 10000000 00010001 01110111

Computer Networks (CSCI 5550/ CSCI 4550/ ECE 4245)

Exam- 2 (V-2)

MCQS and True or False

Time : 15 mins, Marks: 20

Name:__________________                                    Banner  Id:_________________

- (2 Points) What is the TCP's behavior to a loss event indicated by duplicate ACKs?

a. TCP terminates the connection.

b. TCP doubles the value of cwnd.

c. TCP halves the value of cwnd and enters the fast-recovery state.

d. TCP waits for the retransmission timeout to expire before taking any action.

2.  (2 Points)  Move What is the value of ssthresh set to in fast recovery?

a. Same as the value of cwnd

b. Twice the value of cwnd

c.  Twice the value of cwnd when the loss event occurred

d.  Half the value of cwnd when the triple duplicate ACKs were received

3.  (2 Points) DNS What is the purpose of TCP's flow-control service?

- To increase the speed of data transmission

- To match the rate at which the sender is sending with the rate at which the receiver is reading

- To prevent out-of-order segments

- To reduce network congestion

4. (5 points) What is the purpose of error detection in ARQ protocols?

a) To allow the sender to learn of the receiver's view of the world.

b) To enable the receiver to detect and possibly correct packet bit errors.

c) To create extra bits that will be gathered into the packet checksum field of the data packet.

d) To send ACK and NAK packets back from the receiver to the sender.

5. (2 points) What is the service model offered by TCP to Internet applications?

a) A reliable channel through which data can be transferred.

b) An unreliable channel through which data can be transferred.

c) A channel with no guarantee of order or completeness of delivery.

d) A secure channel with encryption.

True/False:

6.   (2 points ) The solution to the performance problem of stop-and-wait behavior is pipelining.

7.  (2 points ) UDP uses the Internet checksum field to detect packet bit errors

8.  (2 points ) Will GBN retransmit all subsequent packets if the acknowledgement for packet n gets lost?

9.  (2 points ) The size of the TCP rwnd never changes throughout the duration of the connection.

10.  (2 points) ARQ is a technique used to ensure reliable delivery of data over an unreliable communication channel.